STEP 1 — USB 연결 (WSL 터미널)

sudo modprobe gs_usb

sudo modprobe vhci-hcd

STEP 2 — USB 연결 (PowerShell 관리자)

usbipd bind --busid 1-1

usbipd attach --wsl --busid 1-1

STEP 3 — CAN 활성화 (WSL 터미널)

cd ~/piper_ws

bash can_activate.sh can0 1000000

STEP 4 — ROS2 노드 실행 (터미널 1)

cd ~/piper_ws

source install/setup.bash

ros2 launch piper start_single_piper.launch.py

(터미널 1은 계속 켜놔야 함)

STEP 5 — RViz 실행 (터미널 2, 새 터미널)

bashcd ~/piper_ws

source install/setup.bash

ros2 launch piper start_single_piper_rviz.launch.py

로봇팔 제어 방법

RViz로 제어

- 슬라이더로 각 조인트 각도 조절

터미널로 제어 (터미널 3, 새 터미널)

bash

cd ~/piper_ws

source install/setup.bash

활성화

bash

ros2 topic pub /enable_flag std_msgs/msg/Bool "data: true"

조인트 제어

bash

ros2 topic pub /joint_states sensor_msgs/msg/JointState "{header: {stamp: {sec: 0, nanosec: 0}, frame_id: 'piper_single'}, name: ['joint1', 'joint2','joint3','joint4','joint5','joint6','joint7'], position: [0.3,0,0,0,0,0,0], velocity: [0.5,0,0,0,0,0,0], effort: [0,0,0,0,0,0,0]}"

Python SDK로 제어

bash

cd ~/piper_ws/src/piper_sdk/scripts

python3 1-1.piper_MoveJ.py

긴급 정지

bash

cd ~/piper_ws/src/piper_sdk/scripts

python3 0-2.piper_emergency_stop.py

긴급 정지 후 복구:

bash

python3 0-3.piper_emergency_restore.py